Annex 1 to the Tender Documentation

# Technical specification of the subject of the contract

Public tender „Protocol analyzer PCIe gen 4“

# Annex 1:	Technical specification

The subject of performance is delivery of the PCI Express gen 4 protocol analyzer for x16 bus width. A bus probe (“slot interposer”) for bidirectional capture of traffic between the host system’s x16 CEM slot and an add-in card with all other necessary supplements (for example interconnect and power cables) must be included. Any software required to work with the device must be also included with a time-unlimited license.

The whole analyzer system must meet all of the following requirements:

- lossless bi-directional capture of data traffic flowing through the PCIe x16 slot for all conceivable data rates according to PCI Express specification, revision 4.0 (2.5 – 16GT/s)

- compatibility with the previous PCIe protocol specs

- size of the capture buffer 8 GB or more

- decode and display of captured transactions with the ability to filter and search

- support for spread-spectrum bus clocking

- manual or programmable (triggered) recording

- external trigger input

- capture triggers must support all of following:

            - invalid symbol, training set, negative acknowledgments (NAK), read error

            - packet or a sequence (ordered set) of given type and content

            - sequence of link or packet events with at least four states

- capture storage qualify filters

- tracking of flow control credits